BF-0712 backfill decision (DQ-3318)

To: data governance board    From: data owner, published demand series    Date: 6 October 2026

Decision: run BF-0712 with 21 incidents removed

Run BF-0712 with 21 of its 38 incidents removed. The amended job backfills 17 incidents covering 602 BA-hours and changes published demand by +496.7 GWh (496,736 MWh). As submitted, the job would have backfilled all 38 incidents (1,408 BA-hours) and changed published demand by -1,117.3 GWh (-1,117,283 MWh). Running it as submitted would have deleted more than a terawatt-hour of load that was really served. Rejecting it outright would leave 17 incidents of demand values that are plainly wrong in the published series.

Why the job cannot run as submitted. The job treats every large gap between reported demand and the day-ahead forecast as bad demand. In 21 incidents the reported demand is right. 17 are forecast errors (613 BA-hours; the job would have changed demand by -1,411.5 GWh), and 4 are real changes in load (193 BA-hours; -202.5 GWh). DQ-STD-007 section 2 rules out backfilling either.

How each incident was decided

A BA's reported demand is derived from its metered net generation minus its total interchange (EIA-930 documentation). So a demand value is wrong when one of its inputs is wrong, or when the demand field itself is broken.

For every incident I compared reported demand, the day-ahead forecast, net generation and total interchange with the same BA's previous seven days. I also rebuilt the BA's total interchange from what each directly interconnected neighbour reported for its side of the tie (EIA-930 INTERCHANGE files; the BA's own figure is used only where a neighbour is silent).

Backfill when demand is reported as zero or negative for a retail BA, or when one input to demand (generation or interchange) departs from the rest of the evidence: interchange contradicted by the neighbours, or generation collapsing while neighbour-confirmed interchange stays put. A real loss of local generation would be met by more imports, not by a matching fall in load.

Leave as reported when demand reconciles with generation and neighbour-confirmed interchange and moves the way real load moves: storm outages with gradual restoration, cold or hot weather. Also leave it when the forecast is the outlier against its own history (stuck, collapsed, or running persistently low) while demand matches its prior week.

EIA's imputation flags do not settle these incidents. EIA imputes demand only when it is blank, zero, negative or at least 1.5 times the BA's past maximum, and it imputed just 36 of the 602 hours backfilled here.

Incident by incident

| Incident | BA, first hour (UTC), hours | Verdict | Submitted / applied (MWh) | Demand vs prior week; forecast; interchange own vs neighbours (MW, medians) | What settles it |
| --- | --- | --- | --- | --- | --- |
| 01 | CPLW, 2024-09-27 11:00, 66h | Leave (real load) | 21,022 / 0 | D 86 vs 518; DF 423; NG 66 vs 329; TI -22 / -22 | Hurricane Helene outage: demand, generation and neighbour-confirmed interchange fall together, then recover over days |
| 02 | IID, 2024-10-12 08:00, 24h | Backfill | 11,765 / 11,765 | D -0 vs 549; DF 456; NG 98 vs 626; TI 467 / 124 | demand reported at about zero while the BA's own generation and interchange show load being served |
| 03 | IID, 2024-11-03 08:00, 25h | Backfill | 6,885 / 6,885 | D 0 vs 324; DF 264; NG 0 vs 498; TI 0 / 265 | demand, generation and interchange all reported as zero for a full day while neighbours report flows into IID |
| 04 | LDWP, 2024-11-17 04:00, 28h | Backfill | 45,642 / 45,642 | D 730 vs 2,378; DF 2,272; NG 324 vs 2,078; TI -448 / -448 | generation reading collapses while interchange (confirmed by neighbours) is unchanged, so demand is understated |
| 05 | IID, 2024-11-22 09:00, 24h | Backfill | 7,125 / 7,125 | D 0 vs 292; DF 301; NG 0 vs 460; TI 0 / 312 | demand, generation and interchange all reported as zero while neighbours report flows into IID |
| 06 | IID, 2024-12-20 09:00, 24h | Backfill | 7,389 / 7,389 | D 0 vs 308; DF 314; NG 0 vs 504; TI 0 / 274 | demand, generation and interchange all reported as zero while neighbours report flows into IID |
| 07 | WAUW, 2025-02-10 19:00, 53h | Leave (forecast error) | -2,335 / 0 | D 124 vs 118; DF 82; NG 63 vs 63; TI -62 / -62 | demand in line with its prior week and neighbour-confirmed; the day-ahead forecast is the outlier |
| 08 | IID, 2025-02-26 09:00, 24h | Backfill | 8,585 / 8,585 | D 0 vs 302; DF 340; NG 0 vs 592; TI 0 / 271 | demand, generation and interchange all reported as zero while neighbours report flows into IID |
| 09 | IID, 2025-04-25 08:00, 24h | Backfill | 8,120 / 8,120 | D 0 vs 332; DF 332; NG 0 vs 734; TI 0 / 400 | demand, generation and interchange all reported as zero while neighbours report flows into IID |
| 10 | WAUW, 2025-07-01 17:00, 37h | Leave (real load) | -1,846 / 0 | D 137 vs 100; DF 87; NG 54 vs 54; TI -82 / -83 | demand above its prior week with neighbour-confirmed extra imports; consistent load, forecast too low |
| 11 | IID, 2025-07-07 08:00, 24h | Backfill | 16,920 / 16,920 | D 0 vs 640; DF 694; NG 914 vs 915; TI 218 / 218 | demand reported as zero while generation and interchange stay at normal levels |
| 12 | NEVP, 2025-07-25 08:00, 24h | Leave (forecast error) | -116,555 / 0 | D 5,528 vs 6,016; DF 692 (one value all incident); NG 4,753 vs 4,788; TI -589 / -589 | forecast stuck at 692 MW for the whole day; demand in line with its prior week |
| 13 | IID, 2025-07-27 08:00, 24h | Backfill | 13,543 / 13,543 | D 0 vs 568; DF 550; NG 0 vs 875; TI 0 / 272 | demand, generation and interchange all reported as zero while neighbours report flows into IID |
| 14 | NEVP, 2025-09-10 08:00, 24h | Leave (forecast error) | -101,213 / 0 | D 4,872 vs 5,452; DF 692 (one value all incident); NG 4,382 vs 5,010; TI -516 / -516 | forecast stuck at 692 MW for the whole day; demand in line with its prior week |
| 15 | IID, 2025-09-16 08:00, 24h | Backfill | 13,827 / 13,827 | D 0 vs 491; DF 561; NG 0 vs 769; TI 244 / 244 | demand and generation reported as zero while interchange shows normal imports |
| 16 | WAUW, 2025-12-17 23:00, 131h | Backfill | 7,191 / 7,191 | D 43 vs 102; DF 97; NG 2 vs 41; TI -40 / -40 | generation reading drops to about 2 MW while interchange is unchanged, so demand is understated |
| 17 | TIDC, 2025-12-29 09:00, 24h | Backfill | 7,442 / 7,442 | D 0 vs 268; DF 323; NG 250 vs 227; TI -64 / -64 | demand reported as zero while generation and interchange stay at normal levels |
| 18 | LDWP, 2026-01-01 09:00, 96h | Backfill | 207,224 / 207,224 | D 132 vs 2,706; DF 2,299; NG 132 vs 1,821; TI 0 / -1,104 | interchange reported as exactly zero for four days while neighbours report about 1,100 MW of imports; generation also collapses |
| 19 | DUK, 2026-01-31 17:00, 27h | Leave (real load) | -184,079 / 0 | D 19,833 vs 16,916; DF 12,680; NG 20,209 vs 16,330; TI 601 / 630 | cold-weather demand above its prior week with generation up and neighbour-confirmed interchange; forecast too low |
| 20 | FMPP, 2026-02-05 16:00, 43h | Leave (forecast error) | -34,499 / 0 | D 2,179 vs 2,230; DF 1,403; NG 2,230 vs 2,163; TI 26 / 18 | demand in line with its prior week and neighbour-confirmed; forecast running about 30% low |
| 21 | FMPP, 2026-03-01 01:00, 34h | Leave (forecast error) | -17,940 / 0 | D 1,693 vs 1,936; DF 1,156; NG 1,508 vs 1,695; TI -104 / -108 | demand in line with its prior week and neighbour-confirmed; forecast running about 30% low |
| 22 | FMPP, 2026-03-06 20:00, 25h | Leave (forecast error) | -16,370 / 0 | D 2,129 vs 1,904; DF 1,466; NG 1,833 vs 1,669; TI -296 / -304 | demand in line with its prior week and neighbour-confirmed; forecast running about 30% low |
| 23 | FMPP, 2026-03-14 14:00, 32h | Leave (forecast error) | -21,685 / 0 | D 2,122 vs 2,088; DF 1,391; NG 1,902 vs 1,818; TI -180 / -187 | demand in line with its prior week and neighbour-confirmed; forecast running about 30% low |
| 24 | FMPP, 2026-03-20 13:00, 27h | Leave (forecast error) | -14,391 / 0 | D 1,769 vs 1,826; DF 1,204; NG 1,490 vs 1,678; TI -133 / -141 | demand in line with its prior week and neighbour-confirmed; forecast running about 30% low |
| 25 | WALC, 2026-03-20 15:00, 63h | Leave (real load) | -37,625 / 0 | D 1,205 vs 880; DF 659; NG 777 vs 799; TI -422 / -420 | demand above its prior week with neighbour-confirmed extra imports and steady generation; forecast too low |
| 26 | FMPP, 2026-03-28 18:00, 64h | Leave (forecast error) | -42,570 / 0 | D 2,002 vs 1,905; DF 1,294; NG 1,860 vs 1,746; TI -78 / -81 | demand in line with its prior week and neighbour-confirmed; forecast running about 30% low |
| 27 | FMPP, 2026-04-05 14:00, 43h | Leave (forecast error) | -29,998 / 0 | D 2,191 vs 2,044; DF 1,500; NG 2,110 vs 1,851; TI -117 / -126 | demand in line with its prior week and neighbour-confirmed; forecast running about 30% low |
| 28 | FMPP, 2026-04-07 23:00, 59h | Leave (forecast error) | -34,842 / 0 | D 1,906 vs 2,034; DF 1,283; NG 1,838 vs 1,928; TI -74 / -79 | demand in line with its prior week and neighbour-confirmed; forecast running about 30% low |
| 29 | SWPP, 2026-04-16 06:00, 24h | Leave (forecast error) | -636,189 / 0 | D 32,988 vs 32,077; DF 5,320; NG 32,484 vs 31,980; TI 274 / 464 | forecast collapses to a few thousand MW for a day; demand, generation and interchange normal |
| 30 | LDWP, 2026-04-18 08:00, 24h | Backfill | 49,475 / 49,475 | D 160 vs 2,788; DF 2,122; NG 160 vs 1,668; TI 0 / -433 | interchange reported as exactly zero while neighbours report about 430 MW of imports; generation also collapses |
| 31 | FMPP, 2026-04-22 22:00, 36h | Leave (forecast error) | -21,674 / 0 | D 1,954 vs 2,009; DF 1,296; NG 1,742 vs 1,830; TI -118 / -126 | demand in line with its prior week and neighbour-confirmed; forecast running about 30% low |
| 32 | SRP, 2026-05-04 21:00, 34h | Backfill | 59,629 / 59,629 | D 2,218 vs 3,236; DF 3,942; NG 5,102 vs 5,034; TI 3,183 / 1,596 | own interchange shows exports about 1,600 MW above what neighbours report, so demand is understated |
| 33 | NEVP, 2026-05-09 08:00, 24h | Leave (forecast error) | -107,826 / 0 | D 5,154 vs 4,364; DF 692 (one value all incident); NG 4,519 vs 4,160; TI -890 / -888 | forecast stuck at 692 MW for the whole day; demand in line with its prior week |
| 34 | IID, 2026-05-20 08:00, 24h | Backfill | 11,704 / 11,704 | D -24 vs 417; DF 442; NG 0 vs 716; TI 8 / 293 | demand reported negative or zero with generation at zero while neighbours report flows into IID |
| 35 | FMPP, 2026-05-31 14:00, 27h | Leave (forecast error) | -23,081 / 0 | D 2,790 vs 2,602; DF 1,834; NG 2,733 vs 2,638; TI 3 / 8 | demand in line with its prior week and neighbour-confirmed; forecast running about 30% low |
| 36 | FMPP, 2026-06-13 15:00, 50h | Leave (forecast error) | -44,597 / 0 | D 2,796 vs 2,515; DF 1,898; NG 2,709 vs 2,452; TI -88 / -106 | demand in line with its prior week and neighbour-confirmed; forecast running about 30% low |
| 37 | NEVP, 2026-06-18 08:00, 24h | Leave (forecast error) | -145,726 / 0 | D 6,864 vs 6,536; DF 692 (one value all incident); NG 5,998 vs 5,852; TI -931 / -863 | forecast stuck at 692 MW for the whole day; demand in line with its prior week |
| 38 | IID, 2026-06-22 08:00, 24h | Backfill | 14,270 / 14,270 | D 0 vs 588; DF 586; NG 866 vs 874; TI 343 / 343 | demand reported as zero while generation and interchange stay at normal levels |

Why no other decision survives

Running the job as submitted fails DQ-STD-007 section 2 on 21 incidents. Rejecting it fails the purpose of the standard on 17 incidents where the published demand is plainly wrong (whole days of zero load at IID and TIDC; LDWP interchange reported as zero while its neighbours report about 1,100 MW of imports into it). Removing incidents is the only option that leaves every reported value that matches the evidence and replaces every one that does not. The largest single item, SWPP on 16 April 2026 (-636,189 MWh submitted), is a broken forecast, not broken demand. The three closest calls are LDWP from 17 November 2024, WAUW from 17 December 2025 and SRP from 4 May 2026. All three are backfilled: in each one input to demand departs from the evidence while the others hold (two generation readings that collapse with interchange unchanged, and one interchange reading that its neighbours contradict).